SENTINEL

Code Explanation Script

Exactly what to say when asked to explain your code

| RACHANA = Blue | PERSON 2 = Green | PERSON 3 = Purple | ⟶ CUE = Action |
| --- | --- | --- | --- |

OPENING — How to Start Any Code Explanation

📝 Use this opening whenever an evaluator says 'explain your code' or 'walk me through the code'.

PERSON 2: Our project has five code files. I'll give you a quick overview and then we can go deep on any specific part you'd like.

PERSON 2: server.py — this is the brain. Python Flask server running on the laptop. Handles face recognition, database, QR verification, SMS, and the admin API.

RACHANA: WROOM_v7.ino — this is the door controller. Arduino C++ running on the ESP32. Handles keypad, RFID, OLED display, buzzer, and the ultrasonic sensor.

PERSON 3: ESP32_CAM_v4.ino — this is the camera node. Takes photos on command and sends them to server.py for processing.

PERSON 2: capture_faces_v3.py — enrollment tool. Captures training photos for new students using the ESP32-CAM directly.

PERSON 3: App.jsx — the React admin dashboard. Shows live logs, anomalies, absentees, and lets admin generate hall tickets.

PERSON 2: Which file would you like us to explain first?

SERVER.PY — Complete Code Walkthrough

📝 Person 2 leads. Others support when their area comes up. Point to the code on screen as you explain.

## 1. Imports — What and Why

PERSON 2: Let me start with the imports — each one has a specific purpose.

⟶ Point to import section on screen

PERSON 2: os and io — file system operations and in-memory buffers. We use os.makedirs to create the alarm_photos and faces folders automatically on startup. io.BytesIO creates an in-memory buffer for QR PNG images so we can send them over HTTP without saving to disk.

PERSON 2: json and hashlib — JSON encoding for all our API responses, and hashlib for SHA256 cryptographic hashing used in QR tamper detection and PIN generation.

PERSON 2: sqlite3 is built into Python — no installation needed. We use it for all database operations. We chose SQLite because it's serverless and file-based — perfect for a single-hall deployment where we don't need a separate database server.

PERSON 2: threading — we run three things simultaneously: the MQTT broker loop in a background thread, SMS sending in a background thread so faculty auth doesn't wait for the SMS, and the webcam QR scanner in a background thread so Flask can keep serving HTTP requests while the camera is open.

PERSON 2: numpy and cv2 — numpy arrays are how OpenCV and DeepFace represent images internally. cv2 is OpenCV for computer vision — face detection, image decoding, QR code scanning.

PERSON 2: Flask, flask_cors — Flask turns Python functions into HTTP endpoints with just a decorator. flask_cors adds cross-origin headers so our React dashboard running on port 5173 can call our Flask API on port 3000 — browsers block this by default without CORS.

PERSON 2: paho.mqtt.client — the MQTT client library. Connects to our Mosquitto broker to receive qr_scan commands from the WROOM and publish results back.

PERSON 2: requests — HTTP client, named req_lib to avoid conflict with Flask's own request object. Used only for Fast2SMS API calls.

## 2. Configuration

⟶ Point to CONFIG section

PERSON 2: These are the key constants at the top of the file.

DEEPFACE_MODEL     = "ArcFace"

DEEPFACE_METRIC    = "cosine"

DEEPFACE_THRESHOLD = 0.40

PERSON 2: ArcFace is our face recognition model — trained on 5.8 million faces, 99.4% accuracy on the LFW benchmark. We use cosine distance which measures the angle between two 512-dimensional face vectors in mathematical space. Zero means identical faces, one means completely different. Our threshold of 0.40 means faces must be at least 60% similar — we chose this stricter value instead of the default 0.68 for exam security. In our tests, Rachana's face scored 0.16 — well within the threshold.

QR_SECRET  = "examhall_qr_secret_2026!!"

SAFE_PIN_DIGITS = [1, 2, 4, 5, 7, 8]

PERSON 2: QR_SECRET is used in SHA256 hashing to make QR codes tamper-proof. SAFE_PIN_DIGITS — we only allow these digits in PINs because digits 3, 6, and 9 are reserved for keypad functions on the WROOM. 3 clears PIN entry, 6 triggers exit mode, 9 tests the buzzer. If these were in a PIN, pressing them mid-entry would accidentally trigger a system function.

## 3. Database Setup

⟶ Point to get_db() and SCHEMA

PERSON 2: Our database has 7 tables. Let me explain the key design decisions.

def get_db():

conn = sqlite3.connect(DB_PATH)

conn.row_factory = sqlite3.Row

conn.execute("PRAGMA foreign_keys = ON")

return conn

PERSON 2: We create a new connection per request rather than sharing one — SQLite connections are not thread-safe and Flask handles multiple requests simultaneously. row_factory = sqlite3.Row makes query results accessible by column name: row['name'] instead of row[0] — much more readable. PRAGMA foreign_keys enforces referential integrity — prevents orphaned records.

CREATE TABLE IF NOT EXISTS access_logs (

auth_method     TEXT,

auth_status     TEXT NOT NULL,

fallbacks_used  INTEGER DEFAULT 0,

entry_time      TEXT,

exit_time       TEXT,

photo_path      TEXT

)

PERSON 2: The access_logs table is the most important for demonstrating production-grade design. auth_method tells us exactly how the student entered — rfid+face, student_pin, or qr_webcam. fallbacks_used tracks how many fallbacks were needed. photo_path stores the face photo as evidence. exit_time is NULL until exit is recorded — then updated in place.

## 4. PIN Generation

⟶ Point to generate_student_pin function

PERSON 2: This is an elegant function worth explaining in detail.

def generate_student_pin(session_id: int) -> str:

raw = f"{session_id}:{PIN_SECRET}:{date.today().isoformat()}"

hash_val = int(hashlib.sha256(raw.encode()).hexdigest(), 16)

for _ in range(6):

pin += str(SAFE_PIN_DIGITS[temp % len(SAFE_PIN_DIGITS)])

temp //= 10

return pin

PERSON 2: We combine the session ID, a secret key, and today's date, then SHA256 hash it. This gives us a large integer. We extract 6 digits by repeatedly taking modulo 6 — the index into our safe digits list — and dividing by 10 to get the next digit. The key insight is this is deterministic — same session ID and same date always produces the same PIN. So if server.py restarts mid-exam, the PIN doesn't change. But different date means different PIN automatically — no manual management needed.

## 5. DeepFace Recognition — Core Function

⟶ Point to verify_face_deepface function

PERSON 2: This is the most important function in the entire system. Let me walk through it.

def verify_face_deepface(img_bytes: bytes):

# Save incoming JPEG to temp file

tmp_path = os.path.join(SNAPSHOTS_DIR, f"_tmp_{ts}.jpg")

with open(tmp_path, "wb") as f: f.write(img_bytes)

PERSON 2: DeepFace requires file paths, not byte arrays. So we save the incoming JPEG bytes to a temporary file, use it, then delete it. The timestamp in the filename prevents collisions when multiple requests come in simultaneously.

detected = face_cascade.detectMultiScale(gray, 1.1, 4, minSize=(40,40))

if len(detected) == 0:

return False, "", 1.0  # strict reject

PERSON 2: Before running the expensive DeepFace model, we first check if a face is actually in the frame using a Haar cascade classifier. If no face detected — camera pointing wrong way, bad lighting, person moved — we reject immediately. This is our strict mode fix. It prevents the system from running deep learning on an empty frame and potentially matching noise patterns.

for roll_no in sorted(os.listdir(FACES_DIR)):

result = DeepFace.verify(tmp_path, ref_photo,

model_name="ArcFace", enforce_detection=False)

distance = result["distance"]

if distance < best_distance:

best_distance = distance

best_roll = roll_no

PERSON 2: We loop through EVERY enrolled student and compare the captured image against their reference photo. The student with the lowest cosine distance wins — if that distance is below 0.40, it's a match. This is identification, not just verification. enforce_detection=False means DeepFace continues even if its own face detector isn't fully confident — important for the ESP32-CAM's lower resolution images.

os.remove(tmp_path)

PERSON 2: We always delete the temp file after recognition. On a long-running server, not deleting temp files would fill up disk space. This is the file system equivalent of a memory leak.

## 6. QR Tamper Detection

⟶ Point to verify_qr function

PERSON 3: Let me explain the QR security model.

payload  = f"UUCMS:{roll_no}:{qr_room}:{exam_start}:{exam_end}"

expected = hashlib.sha256(f"{payload}:{QR_SECRET}".encode()).hexdigest()

if qr_hash != expected: return False, "", False, "tampered"

PERSON 3: When we generate a QR, we create a payload string with all student and exam details, then compute its SHA256 hash using our secret key. This hash is embedded in the QR. On scanning, we recompute the hash from the scanned data. If anyone modifies any field — changes the roll number or room — the hash won't match. SHA256 is a one-way cryptographic function — it's computationally infeasible to forge a valid hash without knowing QR_SECRET.

if row and row["last_used_date"] == today_str:

return False, roll_no, False, "already_used_today"

PERSON 3: One-use-per-day enforcement. After successful scan, we store today's date in qr_tokens.last_used_date. Next scan today returns already_used_today. At midnight it's automatically a different date, so the same QR works again for tomorrow's exam. The student doesn't need a new QR for each exam day.

## 7. Faculty Auth — Idempotency Fix

⟶ Point to faculty_auth function

PERSON 2: This function has an important production-grade fix.

if session["is_active"] and session["student_pin"]:

existing_pin = session["student_pin"]

return jsonify({"success":True, "student_pin":existing_pin})

PERSON 2: If the session is already active — faculty pressed C twice — we return the existing PIN instead of generating a new one. Before this fix, the second press generated a different PIN. Students who had memorized the PIN from the OLED would get rejected. This is called idempotency — calling the same operation multiple times gives the same result. It's a standard production-grade requirement for any API.

## 8. Student PIN Auth — Security Fixes

⟶ Point to auth_pin function

PERSON 2: The emergency PIN endpoint has two important security checks we added.

if student["assigned_room"] != room_id:

return jsonify({"success":False, "reason":"wrong_room"})

PERSON 2: Room check in PIN authentication. Even if a student knows the emergency PIN, they can't use it in the wrong hall. The RFID check happens first anyway, but this adds a second layer.

already_entered = conn.execute("""SELECT id FROM access_logs

WHERE student_id=? AND event_type='entry' AND auth_status='SUCCESS'"""...)

if already_entered: return jsonify({"reason":"already_entered_today"})

PERSON 2: Prevents a student from entering via face recognition, exiting, then re-entering via PIN — which doesn't require being physically present in front of the camera. Once entered successfully today, PIN fallback is blocked for that student.

## 9. Webcam QR Scanner

⟶ Point to webcam_qr_scan function

PERSON 3: The QR scanner deserves special explanation because it's architecturally interesting.

def on_message(client, userdata, msg):

if cmd == "qr_scan":

threading.Thread(target=webcam_qr_scan,

args=(room_id, student_id), daemon=True).start()

PERSON 3: When WROOM sends a qr_scan command via MQTT, server.py intercepts it and starts the webcam scanner in a BACKGROUND THREAD. daemon=True means this thread dies automatically when server.py exits. The ESP32-CAM deliberately ignores qr_scan commands — it used to respond immediately with failure because its fixed-focus lens can't read QR codes reliably, and that failure result was arriving at WROOM before the laptop webcam had even opened. By making the CAM ignore qr_scan, only server.py processes it.

webcam_qr_active  = False

webcam_qr_lock    = threading.Lock()

webcam_qr_pending = []

PERSON 3: Thread safety: if two students trigger QR scan simultaneously, webcam_qr_lock prevents two webcam windows from opening. The second request is queued in webcam_qr_pending. WROOM is told 'scanner busy please wait'. When the first scan finishes, the queued scan starts automatically.

WROOM_v7.ino — Complete Code Walkthrough

📝 Rachana leads — you built this hardware, you know it best.

## 1. Why Arduino C++

RACHANA: We wrote the WROOM firmware in Arduino C++ because the ESP32 microcontroller has only 520 kilobytes of RAM. Python's interpreter alone needs 2 megabytes — it simply won't fit. C++ compiles to native machine code that runs directly on the chip with no interpreter overhead. And real-time keypad scanning needs sub-millisecond response time — only achievable in C++.

## 2. Libraries

⟶ Point to #include section

RACHANA: SPI.h is for the RFID reader — it uses SPI protocol, a 4-wire synchronous communication: clock, data out, data in, and chip select. MFRC522.h is the driver for the specific RFID chip.

RACHANA: Keypad.h handles the 3x4 matrix keypad. It scans by pulling each row LOW and checking which column goes LOW — that tells us exactly which key was pressed. It also handles debouncing automatically, so one physical keypress doesn't register as multiple presses.

RACHANA: Wire.h is for I2C protocol used by the OLED display. Adafruit_SSD1306.h drives the display — handles the 128x64 pixel buffer, text rendering, and display commands.

RACHANA: HTTPClient.h lets the ESP32 send HTTP POST requests to server.py. PubSubClient.h is the MQTT client for pub/sub communication with the ESP32-CAM. ArduinoJson.h handles JSON encoding and decoding for all our message payloads.

## 3. State Machine Architecture

⟶ Point to STATE MACHINE comment and appState variable

RACHANA: The core architecture of the WROOM firmware is a finite state machine. The variable appState can be 0 through 7, each representing a different operational state.

int appState = 0;

// 0=home 1=wait RFID 2=face scan 3=faculty PIN

// 4=student PIN 5=fallback menu 6=QR scan 7=exit

RACHANA: State 0 is home — the system is idle, trespass detection is active, keypad is watching for A, B, or C. State 1 is waiting for RFID tap. State 2 is waiting for face recognition result from the CAM via MQTT. State 3 is faculty PIN entry.

RACHANA: The reason we use a state machine is to prevent race conditions. If the system is in State 2 waiting for a face scan result, it ignores keypad input except B for cancel. If we didn't have states, two things could happen simultaneously — like processing a new RFID tap while already doing a face scan — leading to unpredictable behavior. Finite state machines are the standard architecture for embedded door controllers, ATMs, and traffic lights.

## 4. Non-Blocking Buzzer

⟶ Point to BuzzerPattern struct and buzzerUpdate function

RACHANA: This is one of the most important architectural decisions in the firmware.

struct BuzzerPattern {

int  onMs, offMs, repeats, current;

bool isOn, active;

unsigned long lastChange;

};

RACHANA: This struct stores the complete state of a buzzer beep pattern — how long each beep is, how many repeats, and when the last toggle happened.

void buzzerUpdate() {

if (!buzzer.active) return;

if (buzzer.isOn && now - buzzer.lastChange >= onMs) {

digitalWrite(BUZZER_PIN, LOW);

buzzer.isOn = false;

}

}

RACHANA: buzzerUpdate() is called every loop iteration. It checks if enough time has passed since the last state change using millis() — the milliseconds since boot. If yes, it toggles the buzzer. If no, it returns immediately without blocking anything.

RACHANA: The alternative is delay(500) which would FREEZE the entire microcontroller for 500 milliseconds. During that time, keypad inputs are completely missed, RFID can't be read, and MQTT messages are lost. The millis()-based approach has zero blocking time — everything keeps working while the buzzer beeps.

## 5. RFID Reading

⟶ Point to RFID reading section in State 1

RACHANA: RFID reading happens in State 1.

if (rfid.PICC_IsNewCardPresent() && rfid.PICC_ReadCardSerial()) {

for (byte i = 0; i < rfid.uid.size; i++) {

if (rfid.uid.uidByte[i] < 0x10) uid += "0";

uid += String(rfid.uid.uidByte[i], HEX);

}

uid.toUpperCase();

RACHANA: PICC_IsNewCardPresent checks if a card is in the RF field. PICC_ReadCardSerial reads its 4-byte unique ID. The UID is 4 bytes — we convert each to hexadecimal. The check for less than 0x10 adds a leading zero — byte 0x0F becomes '0F' not 'F'. Without this, cards with low-value bytes would produce shorter UIDs that don't match the database. toUpperCase ensures consistent formatting.

## 6. Trespass Detection

⟶ Point to trespass check in main loop

RACHANA: Trespass detection has the most complex logic — let me walk through all the conditions.

if (appState == 0 && !authActive && !buzzer.active &&

millis() > trespassCooldown &&

millis() - lastDistCheck > 600) {

RACHANA: Six conditions must ALL be true: system is idle, no authentication in progress, buzzer not sounding from previous alarm, cooldown period expired, and 600 milliseconds since last sensor read. This multi-condition guard means trespass detection has zero false positives during normal use.

if (trespassConfirmCount == 0 && trespassFirstSeen == 0) {

trespassFirstSeen = millis();

}

if (millis() - trespassFirstSeen < TRESPASS_GRACE_MS) {

// grace period — wait for A keypress

RACHANA: When someone first enters range, we start a 2-second grace period. This gives legitimate students time to press A and start authentication. If they press A within 2 seconds, authActive becomes true, the counter resets, and no alarm fires. Only if they don't press A — they're a trespasser — does counting begin after 2 seconds.

trespassMissCount++;

if (trespassMissCount >= TRESPASS_MISSES_RESET) {

trespassConfirmCount = 0;

}

RACHANA: This fixes the inching loophole. Previously, one out-of-range reading reset the counter — so someone could inch in and out of range forever without triggering. Now they need 3 consecutive out-of-range readings before the counter resets.

## 7. HTTP to server.py

⟶ Point to httpPost function

RACHANA: All communication from WROOM to server.py goes through this single function.

String httpPost(const String& endpoint, const String& body) {

String url = "http://" + String(SERVER_IP) + ":" + String(SERVER_PORT) + endpoint;

http.begin(url);

http.addHeader("Content-Type", "application/json");

int code = http.POST(body);

String resp = (code > 0) ? http.getString() : "{"error":"http_" + String(code) + ""}";

http.end();

return resp;

}

RACHANA: http.begin sets the target URL. addHeader sets Content-Type so server.py knows it's receiving JSON. POST sends the request. Negative return codes are ESP32 errors like WiFi disconnected — we handle those gracefully by returning an error JSON string. http.end releases the HTTP connection resources — critical to call this, otherwise we run out of sockets.

ESP32_CAM_v4.ino — Complete Code Walkthrough

📝 Person 3 leads this section.

## 1. Purpose and Design

PERSON 3: The ESP32-CAM firmware is deliberately simple. It has one job — receive MQTT commands and send photos. All intelligence is in server.py. The CAM has no display, no keypad, no buttons. It just listens and shoots.

## 2. OV3660 Camera Initialization

⟶ Point to initCamera function

PERSON 3: Camera initialization is the most hardware-specific part.

config.pixel_format = PIXFORMAT_JPEG;

config.fb_count     = 2;  // double buffer

PERSON 3: PIXFORMAT_JPEG means the OV3660 sensor compresses images to JPEG directly in hardware — no CPU overhead. A 640x480 face image is about 15 kilobytes as JPEG versus 900 kilobytes as raw RGB — that's a 60x reduction. Double buffering means one frame buffer fills while the other is being transmitted — eliminates frame tearing and reduces latency.

if (s->id.PID == OV3660_PID) {

s->set_vflip(s, 1);       // CRITICAL — OV3660 mounted inverted

s->set_saturation(s, -2); // OV3660 oversaturates by default

s->set_contrast(s, 1);

}

PERSON 3: The OV3660 sensor is physically mounted upside-down on the AI Thinker board. Without vflip=1, DeepFace receives inverted face images and recognition fails. This was a critical discovery — before this fix, face recognition was failing even with correctly captured training photos. OV3660 also oversaturates colors compared to OV2640 — saturation minus 2 corrects this for better grayscale conversion during face detection.

## 3. Resolution Strategy

void setResolution(framesize_t size) {

s->set_framesize(s, size);

for (int i = 0; i < 2; i++) {

camera_fb_t* fb = esp_camera_fb_get();

if (fb) esp_camera_fb_return(fb);

delay(50);

}

}

PERSON 3: After changing resolution, the camera buffer still contains old frames at the previous size. We drain two frames before capturing. Without this drain, the first capture returns a wrong-size image that server.py cannot decode — causing sporadic face recognition failures. The 50 millisecond delay allows the sensor to stabilize.

## 4. Multipart HTTP Upload

⟶ Point to httpPostImage function

PERSON 3: This is the most complex function in the CAM firmware — manual multipart form construction.

String boundary = "SentinelBoundary7734";

String partImageHeader =

"--" + boundary + "\r\n"

"Content-Disposition: form-data; name=\"image\"; filename=\"cam.jpg\"\r\n"

"Content-Type: image/jpeg\r\n\r\n";

PERSON 3: Multipart form-data is the HTTP standard for file uploads. The boundary string separates different form fields. Arduino's HTTP library doesn't support multipart natively, so we construct the raw bytes manually. The format is: boundary marker, then headers, then the JPEG data, then closing boundary.

uint8_t* body = (uint8_t*)malloc(totalLen);

memcpy(body + pos, fb->buf, fb->len);

int code = http.POST(body, totalLen);

free(body);

PERSON 3: malloc allocates heap memory for the combined request body. memcpy copies the JPEG bytes into the correct position. After POST, free releases the memory. This free call is critical — a memory leak here would crash the CAM after a few hundred face scans as the heap fills up.

## 5. Why CAM Ignores QR Scan

} else if (cmd == "qr_scan") {

// server.py handles QR via laptop webcam — CAM ignores

Serial.println("[CMD] qr_scan — handled by server.py");

PERSON 3: This one-line change fixed a race condition that was causing QR scans to always fail. Previously, when WROOM triggered QR scan, both the CAM and server.py received the command. The CAM would immediately try to decode the QR with its fixed-focus lens, fail, and publish no_qr_detected to MQTT. This failure result arrived at WROOM in about 2 seconds — before server.py's laptop webcam had even opened. So WROOM showed QR FAILED and gave up. By making the CAM ignore qr_scan entirely, only server.py handles it via the laptop webcam, which has autofocus and produces much better image quality for QR decoding.

capture_faces_v3.py — Code Walkthrough

📝 Any team member can explain this — it's straightforward.

## 1. Why This Script Exists

PERSON 2: DeepFace needs at least one reference photo per student to compare against during recognition. This script captures those photos. The critical design decision is Mode 2 — capturing via ESP32-CAM instead of the laptop webcam.

PERSON 2: If we train on laptop webcam photos but recognize via OV3660, the two cameras produce different images — different color temperature, lens distortion, JPEG compression artifacts. DeepFace handles this better than LBPH, but still benefits from matched cameras. Mode 2 completely eliminates this mismatch by using the same sensor for both training and recognition.

## 2. Blur Detection

def is_blurry(image):

gray = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)

return cv2.Laplacian(gray, cv2.CV_64F).var() < BLUR_THRESHOLD

PERSON 2: The Laplacian operator is a second-derivative edge detector. Sharp images have strong edges — high variation in the Laplacian output — meaning high variance. Blurry images have weak edges — low variance. We reject any frame below our threshold of 80. This ensures training photos are sharp, which directly improves recognition accuracy.

## 3. ESP32-CAM Capture via MQTT

client.publish("exam/cam/command",

json.dumps({"cmd":"snapshot", "reason":"enrollment"}))

files = glob.glob("alarm_photos/alarm_*.jpg")

latest = max(files, key=os.path.getmtime)

PERSON 2: We trigger an ESP32-CAM snapshot via MQTT — the same command used for trespass snapshots. The CAM saves the photo to alarm_photos/. We find the newest file using glob and getmtime — the modification time. This is reliable even if there are existing photos in the folder because we always take the most recently modified file.

QUICK Q&A — Code-Specific Questions & Answers

📝 Use these when evaluators ask follow-up questions about specific code choices.

Q: Why did you use Flask instead of Django?

A: Flask is a micro-framework — it gives us just routing with minimal overhead. Django is designed for full web applications with templates, ORMs, and admin panels. We only needed HTTP routing to serve the ESP32 boards and the dashboard. Flask was the right tool — simpler, faster to build, less code to maintain.

Q: Why SQLite instead of MySQL or PostgreSQL?

A: SQLite is serverless and file-based — the entire database is a single file called exam.db. No separate database server process, no configuration, no user management. For a single exam hall with up to a few hundred students, SQLite is more than sufficient. If we scaled to 10+ simultaneous halls, we'd migrate to PostgreSQL — the SQL queries are standard and the switch would take a day.

Q: What is cosine distance and why use it for face recognition?

A: ArcFace represents each face as a 512-dimensional vector — think of it as a point in 512-dimensional mathematical space. Cosine distance measures the angle between two such vectors. Zero means the vectors point in exactly the same direction — identical faces. One means they point in opposite directions — completely different people. We use cosine instead of Euclidean distance because it's scale-invariant — it doesn't matter if the vectors are long or short, only their direction matters. This makes it robust to lighting variations.

Q: How does the non-blocking buzzer work exactly?

A: Instead of delay(500) which freezes everything, we use millis() which returns milliseconds since boot. Each loop iteration, buzzerUpdate() checks: has enough time passed since I last toggled the buzzer? If yes, toggle and record the time. If no, return immediately. This takes microseconds regardless of the pattern timing. The keypad, RFID reader, and MQTT processing all continue normally between buzzer state changes.

Q: Why does the CAM ignore qr_scan commands?

A: This fixes a race condition. When WROOM sent qr_scan, both the CAM and server.py received it. The CAM's fixed-focus lens can't read QR codes reliably, so it immediately published a failure result. That failure arrived at WROOM in about 2 seconds — before server.py's laptop webcam had even opened. WROOM showed QR FAILED. By making CAM ignore qr_scan, only server.py processes it via laptop webcam, which has autofocus and reliable QR decoding.

Q: Why do you drain frames after changing camera resolution?

A: The OV3660 camera buffer holds 1-2 frames. After changing resolution, those buffered frames are still at the old size. If you capture immediately, you get a wrong-size JPEG that server.py can't decode. The 2-frame drain with 50ms delay ensures the first captured frame is at the new resolution. Without this, face recognition would fail sporadically — hard to debug because it happened only on the first scan after resolution change.

Q: Why is vflip needed for OV3660?

A: The OV3660 sensor is physically soldered upside-down on the AI Thinker ESP32-CAM board — it's a hardware constraint of the board design. Without set_vflip(s, 1), the captured image is literally upside-down. DeepFace tries to find a face in an inverted image — the Haar cascade and ArcFace both fail on inverted faces. This was a critical bug we discovered during testing when face recognition was failing even with correct training photos.

Q: How does the tamper detection in QR codes work?

A: We use HMAC-style signing with SHA256. The QR payload contains student ID, room, and dates. We append our secret key and hash everything with SHA256. This hash is embedded in the QR. On scanning, we recompute the hash from the scanned fields plus our secret. If anyone modifies any field — changes the room or roll number — the recomputed hash won't match the embedded hash. SHA256 is a one-way function: you cannot work backwards from the hash to find an input that produces it without knowing the secret key.

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

You wrote this code. You debugged it. You know it. Explain it confidently! 🎉